LESSON NOTE

TOPIC: INTRODUCTION TO TEXTILE STUDY

MEANING OF TEXTILES

Textiles are materials made from interlace materials such as fibres, thin threads, filaments etc. There are natural, synthetic or combination of both. Textiles material can be classified in natural fibres and man made fibres. Natural fibers are organic materials while man made fibers are synthetic or industrial materials. There are variety textile fibres products which are found clothing, furniture, bedding and even some car parts.

Textiles are everywhere.

Basic terms used in textile study are as follows.

1. Fibre

2. Yarn

3. Fabric

4. The wrap

5. The weft

6. The selvedge

7. Grain

8. right side

9. The wrong

1. Fibre: This is a hair –like basic unit of raw material used in the making of yarns and fabrics, e.g. Cotton, silk, wool, nylon, fibres.

2. Yarn: This is a thread made by twisting or spinning fibres.

3. Fabric: This is cloth constructed with yarn or directly from fibres by weaving, knitting, crocheting, and felting .A woven fabric is made up of two sets of yarn or thread –the warp and weft.

4. The wrap: This is the yarn or thread which runs length –wise in a woven fabric .It is parallel to the selvedge.

5. The weft: This is the yarn that runs cross –wise in a fabric .It runs at right angles across the selvedge grain.

6. The selvedge: This is the edge of the fabric made by the weft thread or yarn by turning over the wrap thread .It is the mill-finish edge of a fabric which runs in a length –wise direction.

6. Bias: This is the diagonal direction across the two grain lines, warp and weft.

7. True bias: Makes an angle of 450 across the length –wise and across wise grains .It has the greatest stretch.

8. Gran of the fabric: This refers to the direction of yarns or threads in a fabric.

9. The right side (RS) of the fabric is the side to be worn out. It has better finish and more distinct print than the other side.

10. The wrong side (WS)  of the fabric is the side to be worn inside

LESSON NOTE

THE MAJOR REASONS FOR THE STUDY OF TEXTILE INCLUDE:

1. It helps us to identify the various types of fabrics and their properties.

2. It helps us to select or choose the suitable fabric for a given purpose e.g. choosing good quality fabrics suitable for bed sheets, curtains, dresses etc.

3. It helps us with the understanding of how to cut the fabrics along with grains or the bias when designing a fabric.

4. The study of textile helps one to understand the properties of the fabrics that will enable you to choose the best method of washing and dry cleaning. The proper care of fabrics makes them look better and last longer.

5. It helps one to develop interest in clothing and textile career.

6. It helps consumers and the traders to know what to buy and how to advise others who do not know.

THE IMPORTANCE OF TEXTILE TO MAN

Textiles are very important to man because they're used for:

1. Construction of personal and family clothes e.g. dresses underwear etc.

2. Construction of household articles such as curtains, bed sheets, tablemat etc.

3. Construction of sportswear such as shoes, T shirt etc.

4. Making of protective wears e.g. bullet proof vest, apron etc.

5. Making of bandage, plaster, mask, overall.

6. Making swim wears, skier wears etc.

7. Beautifying or adoring the body.

8. Making nets and tents.

Difference between Natural and Synthetic fibres

NATURAL FIBRES                                                                       SYNTHETIC FIBRES

Comes from nature	                                                               Man made fibres

Natural colour	                                                                          Colour as per requirement is added bath

During spinning process spinneret is not necessary	              During spinning process spinneret is

necessary for the production of

filament

Chances of containing dust or impurities	                          No chance of any dust or impurities

Less durable than synthetic	                                                  More durable than natural

LESSON NOTE

TOPIC: PRODUCTION OF TEXTILES

Textile materials are produce by different construction methods. The appearance, properties, uses and care are affected by way they are constructed.

Fabric construction/textile production is the process of twisting fibers into yarns and yarns into fabrics

Fibers =yarns=fabrics

Fabric production or construction methods

Fabrics are produced using different methods. These methods include:

1. Weaving: This is the process of constructing a fabric by interlacing two set of yarns (warp and weft) to form a close fabric.

2. Knitting: This is the process of making a fabric by interloping the yarns or threads, the loops support each other like a chain.

3. Crocheting: This process of making a fabric by chain of loops from a single yarn, using just one hook or needle.

4. Braiding: This is the process of making a fabric by plaiting two or three yarns to form a strip which are sewn together.