LESSON NOTE 10

MAP WORK/READING Map work or map reading involves the ability to recognize conventional signs on a map using the signs to interpret the map. A map is a representation of the earth's surface as it is seen from above on a paper. TYPES OF MAPS 1. General maps: show more than one type of information at a time. Eg Topographic or survey maps 2. Thematic maps: deals with a particular class of information and the theme serves as the title of the map. Eg. Vegetation map of Nigeria 3. Atlas: collection of special maps found in large volumes or in the form of a book, they cover several topics like climate, population, settlement etc. 4. Statistical or distribution maps: shows trend and spread of physical and cultural features over defined geographical areas. 5. Sketch maps: rough maps drawn to illustrate or give explanation on a paper drawn with free hand. SCALES Scale of a map is the relationship between the distance on the map and the actual distance on the ground or the earth's surface. TYPES. Based on size, two types of scale exist; 1. Large scale: the size of a map that covers a small area but show great details. Eg. 1cm represents 100cmm or 100000cm. They are used on big sized paper and smaller denominator means larger area. 2. Small scale: map size that covers a small piece of paper but showing information on large areas such as continents. Example is atlas. They do not show great details. Based of method of expression, 3 types of scales exist 1. Statement scale: by making a direct statement e.g 1cm represents 1km. It is given in words and figures and is the simplest form of expressing scale. 2. Ratio scale or Representative fraction: it expresses scale either in fraction or ratio. E..g RF for a scale 1cm to 2km will be 1÷200,000. The numerator which is always 1 represents length on the map while the denominator represents the actual distance on the ground. 3. Linear scale: indicated by means of a straight line which is conveniently subdivided so that distances on the map can easily be read using a piece of ruler or divider. CONVERSION OF SCALES Converting statement to Rf. 1cm rep 2km 1cm rep 2km in Rf = 1cm:2km =1cm÷2km But 100,000cm =1km 1cm ÷2km = 1cm÷2×100,000cm =1cm÷200,000 or 1:200,000 Converting Rf to statement scale 1÷50,000 Using cm 1÷50,000 in statement = 1cm rep 50,000cm I.e 1cm rep (50,000÷100,000) km = 1÷2km Thus 1÷50,000 in statement is 1cm rep 1÷2km HOME WORK list 4 uses of maps Identify 2 advantages and disadvantages each for statement representative fraction and linear scales.